# 0511-T16-2

# CLAIMS

# 1. A recycling equipment

## for recovering materials by recycling products comprising plastic materials, superabsorbent polymers (SAP), and optionally cellulosic fibers,

### which are loaded with waste materials including human or animal exudates,

## said equipment comprising

### -	a product receiving unit;

### -	optionally a pre-washing unit;

### -	a product shredding unit;

### -	a plastic separation unit;

### -	optionally a SAP fiber separation and fiber recovery unit;

### -	a SAP recovery unit,

## further comprising an overall fluid handling system,

### adapted to minimize, including eliminating, ingoing or outgoing fluid streams,

#### whereby said ingoing or outgoing fluid streams are preferably aqueous fluids,

### and comprising elements for adjusting salt concentrations of liquids for being selectively supplied to and received from said equipment units.

# 2. A process

## for recovering materials by recycling products comprising plastic materials, superabsorbent polymers (SAP), and optionally cellulosic fibers,

### which are loaded with waste materials including human exudates,

## said process comprising the product recycling steps of

### -	receiving used products in a product receiving unit;

### -	optionally pre-washing said products in a a pre-washing unit;

### -	shredding said used products in a product shredding unit;

### -	separating plastic materials in a plastic separation unit;

### -	optionally recovering cellulosic fibers in a SAP /fiber separation and fiber recovery unit;

### recovering SAP in a SAP recovery unit,

## further comprising the step of operating an overall fluid handling system comprising the steps of

#### -	receiving incoming fluids, preferably aqueous liquids;

#### -	preparing at least one salt solution from said incoming fluids comprising predetermined salt concentration and optionally salt composition;

#### -	distributing said salt solutions to said recycling steps;

#### -	discharging spent salt solutions from said recycling steps, whereby said spent salt solutions exhibit differing degree of waste loads;

#### -	treating said spent salt solutions into two or more fractions, whereby one fraction comprising a high waste load is discharged to a waste water treatment preferably a biogas generation unit or a composting unit and at least one further fraction is recycled to said product recycling steps,

#### whereby optionally the high waste load fraction may be reduced to dryness.

# 3. A fluid handling system

## for providing and receiving fluids to and from processing steps for recovering materials from recycled products,

### whereby said products comprise plastic superabsorbent polymers (SAP) and optionally cellulosic fibers,

## said fluid handling system comprising

### -	at least one incoming fluid tank;

### -	at least one spent process fluid tank;

### -	at least one separation unit for separating spent process fluids into

#### - 	a clean fluid fraction, adapted to be reused in said processing step, optionally fed into said incoming fluid tank,

#### -	a loaded fluid fraction for being discharged to a waste water treatment preferably a biogas generation unit or a composting unit, optionally comprising a drying unit for solidifying the loaded fluid fraction.

# 4. An energy management system

## for providing and receiving energy to and from processing steps for recovering materials from recycled products,

### whereby said products comprise plastic superabsorbent polymers (SAP) and optionally cellulosic fibers,

## said energy management system comprising

### -	at least one solar thermal energy collector;

### -	at least one thermal energy storage element;

### -	distribution and control elements for delivering and receiving energy to and from said processing steps.

# 5. A process control system

## for controlling processing steps for recovering materials from recycled products,

### whereby said products comprise plastic superabsorbent polymers (SAP) and optionally cellulosic fibers

## whereby said process control system comprises

### -	process parameter monitoring elements;

### -	process parameter change element that allow change of parameters of said processing steps;

### -	process parameter input elements that allow process parameters to be set to predetermined parameter;

### -	process parameter optimization elements that allow calculation of optimal combination of various process parameter according to predetermined optimization targets.