ABSTRACT

The sidecar wheel is positioned forward of the passenger seat and protrudes from the base of sidecar capsule.

Primary benefit is the suitability for compact size, up to 20% smaller than conventional designs, yet maintaining comfort for the passenger. Repositioning the tire increases width for passengers and is beneficial in Singapore whereby the legal dimensions are 1400mm(Length) x 1020mm(Width) x 800mm(Height). Figure 1 shows the legally allowed limits of a sidecar in Singapore.

Secondary benefits are:

- Lower center of gravity, capsule position is lower relative to tire.

- Improved stability on straights due to increased wheel lead (WL). WL is the distance between sidecar wheel and rear wheel.  A typical WL is 250-350mm. For this design, WL can be from 600mm.

- Better overall handling, as the sidecar is closer to motorcycle.

Figure 1